### Safety Study Guide

## Safety Study Guide

# Basic Lab Safety

Foundations of Technology

# Name:_________________________________ Date:_____/_____/_____

# Instructions:  Read and study the safety rules as presented in the target session.

- Always wear eye protection.

- Use equipment only with the instructor’s permission.

- Use the equipment only when the instructor is in the shop.

- Report all accidents or injuries immediately no matter how small.

- Do not use defective equipment.

- Never use equipment until you have been properly instructed in its use.

- Loose clothing can cause injuries.

- Long hair must be tied back in a bun (not pony tail).

- No jewelry should be worn in the shop.

- Wear closed toe shoes or boots.

- A dust mask should be worn when working where there is a lot of wood dust.

- Finishing materials should only be used in well ventilated areas.

- Always clean up spilled materials.

- Used rags are to be kept in a ventilated metal container.

- Keep cabinet doors and drawers closed.

- Never leave anything on the floor that could be slipped on or tripped over.

- Close vises when they are not in use.

- Don’t distract someone while they are using the equipment.

- Never fool around or play in the shop.

- Carry sharp tools with the edge or point held down toward the floor.

## Safety Study Guide

### Safety Study Guide

# Hand Tool Safety

Production Technology

# Instructions:  Read and study the safety rules as presented in the target session.

- Always wear eye protection when working in the shop.

- Use only the type of tools that are designed for the work you are doing.

- Always use the right size tool for the job

- Check tools to be sure they are sharp, clean and in good condition before you use them.

- Don’t use a tool that is dull, broken or damaged.

- Tools should always be carried by the handle.

- Carry sharp tools so the edge or the points is aimed down toward the floor.

- The material should be fastened down so that both hands are left free to use the tools.

- Never lay tools down near the edge of the bench where they could roll off, or where someone could bump into them.

- Never aim a cutting tool toward yourself or toward anyone else.

- Keep your hands and arms behind the cutting edge of the tool.

- Tools that are not in use should be returned to the assigned storage area.

- Tools should be used only if you understand how to use them correctly and safely.

### Safety Study Guide

## Safety Study Guide

# Drill Press Safety

Production Technology

# Instructions:  Read and study the safety rules as presented in the target session.

- Long hair must be tied back in a bun when you operate the drill press.

- Use only bits that are designed and manufactured for machine operation.

- Be sure the bit is sharp, straight and in good condition.

- Set the drill speed for the type of cut that is going to be made.

- The clearance hole in the center of the table must be lined up directly under the cutting tool.

- Use clamps to hold the material firmly in the table.

- Use the chuck key to tighten the chuck.

- Check twice to be sure the chuck key has been removed from the chuck before you turn on the drill press.

- Keep your hands and fingers away from the area directly under the drill bit.

- Back out of a deep cut from time to time to prevent the bit from clogging and over-heating.

- A backer board should be used under the material when a hole is to be drilled all the way through the stock.

- The drill press must be completely stopped before changes or adjustments are made.

- Let the drill press coast to a stop.  Never try to stop it by hand.

- Use a brush to clear waste material from the drill press table or blow it away with your mouth.

- Never leave the drill press while it is still running.  Wait for it to come to a complete stop before you leave it.

- Get the instructor’s permission before you use any special set-ups or attachments.

### Safety Study Guide

## Safety Study Guide

# Bandsaw Safety

Production Technology

# Instructions:  Read and study the safety rules as presented in the target session.

- The wheel guards must be closed and locked before the saw is turned on.

- The blade guide should be adjusted so that it is about ¼ inch above the material you are going to cut.

- When you switch on the saw, make sure it is tracking properly.

- If the saw makes a clicking noise while it is running, turn it off immediately.

- Never stand on the right side of the table, and don't let anyone else stand on that side while the saw is running.

- If the blade breaks, get out of the way immediately.

- Never make adjustments on the saw while it is running.

- Make sure the material is clear of the blade.

- Wait for the saw to come up to full speed before you start the cut.

- Feed the material smoothly and evenly through the cut.  Don't force it.

- Do not back out of a cut while the saw is running.  Stop the saw and wait for it to come to a complete stop.

- A narrow blade should be used for cutting small circles or shape curves.  Relief cuts should be made first.

- Guide the material so your hands and fingers are never in the path of the cut.  Keep them out of the line of the cut.

- Use a piece of wood to remove scraps from around the blade.

- The material must be held down firmly against the table while it is being cut.

- Plan the cut so you can keep your balance all the way through the cut.

- You should never try to cut cylindrical stock on the bandsaw.

- Always turn the bandsaw off and wait for it to come to a complete stop before you leave it.

- Do not use the bandsaw until you understand how to use it correctly and safely.